FITTING INSTRUCTION

| Clamp mark in acc. with |  | # Cables joining |
| --- | --- | --- |
| ISO | PN |  |
| 1 | L | Left directional lights |
| 2 | + | Rear fog lights |
| 3 | 31 | Ground |
| 4 | R | Right directional lights |
| 5 | 58R | Right side parking lights |
| 6 | 54 | Stoplights |
| 7 | 58L | Left side parking lights |

1

2

3

4

5

6

M12x25

M12x35

M12x35

M10x35

M10x35

Washer

ø35xø12x3

18

A

A

B

C

## This towbar is designed to assembly in following cars: 
FORD FOCUS, 3-5 doors,	produced since 1998 till 2004, catalogue no. C27A and is prepared to tow trailers max total weight 1400 kg and max vertical mass 50 kg.

From manufacturer

Thank you for buying our product. Their reliability has been confirmed in many tests. Reliability of towbar depends also on correct assembly and right operation. For this reasons we kindly ask to read carefully this instruction and apply to hints.

The towbar should be install in points described by a car producer.

# The instruction of the assembly

- Disassemble the rear bumper and then unscrew the metal profile. This profile will not be reassemble.

- Lower  the silencer from the rubber handle and remove the thermal silencer protection.

- Put side brackets (pos. 4 and 5) to chassis members and then fix loosely through holes (pos. A) using bolts M10x35mm (pos. 9).

Attention! Use jibs with nuts (pos. 6).

- Fix loosely through holes (pos. B) using M10x35mm (pos. 9), but through holes (pos. C) fix using nuts M8 (use from the unscrewed metal profile).

- Position the main bar of the towbar (pos. 1) between mounted brackets (pos. 4 and 5) and fix with M12x35mm (pos. 8) from the towbar accessories.

- Reassemble the rear bumper

- Fix body of the automat (pos. 18) and the socket plate (pos. 3) using bolts M12x25mm (pos. 7) from accessories. Place tow-ball (pos. 2) according to supplied instruction.

- Tighten all nuts and bolts according to the torque shown in the table.

- Connect the electric wires according to the instructions of the car.

- Complete the paint cover of towbar (during the mounting paint cover could be destroyed).

Torque settings for nuts and bolts (8,8):

M 8 - 25 Nm		M 10 - 55 Nm

M 12 - 85 Nm		M 14 - 135 Nm

NOTE

After install the towbar you should get adequate note in registration book (at authorised service station).The car should be equipped with:

- Indicators

- Tow mirrors

After 1000km of exploitation check all bolts and nuts. The ball of towbar must be always kept clear and conserve with a grease.

Towbar accessories:

PPUH   AUTO-HAK S.J.

Produkcja Zaczepów Kulowych

Henryk & Zbigniew Nejman

76-200 SŁUPSK  ul. Słoneczna  16K

tel/fax  (059) 8-414-414; 8-414-413

E-mail: office@autohak.com.pl

www.autohak.com.pl

Towing hitch (without electrical set)

Class: A50-X        Cat. no. C27A

Designed for:

Manufacturer: FORD

Model: FOCUS

Type: 3-5 doors

produced since 1998 till 2004

Approval number according to Directive 94/20/EC: e20*94/20*0924*00

Technical data:

D-value: 7,55 kN

maximum trailer weight: 1400 kg

maximum vertical cup mass: 50 kg

Foreword

This towbar is designed according to rules of safety traffic regulations. The towing hitch is a safety component and can be install only by qualified personnel. Any alteration or conversion of the towing hitch is prohibited and would lead to cancellation of design certification. Remove insulating compound and underseal from vehicle (if present) in the area of the matting surfaces of the towing hitch.

The vehicle manufacturer’s specifications regarding trailer load and max. vertical cup load are decisive for driving whereat values for the towing hitch cannot be exceeded.

D-value formula:

Max trailer weight [kg]    x

Max trailer weight [kg]   +

Max vehicle weight [kg]

Max vehicle weight [kg]

X

9,81

1000

=

D [kN]